______________________________________________________________

RTL8812AU Efuse Contents

Date: 2013/02/06

Version: 3.8

This document is subject to change without notice. The document contains Realtek confidential information and must not be disclosed to any third party without appropriate NDA.

# Efuse Contents

The RTL8812AU is embedded an internal 1024-byte non-volatile memory called eFuse. The definition of each Efuse map is shown as the below.

Note: It is suggested to obtain Realtek approval before any change on the default settings of the Efuse.Table 1. Efuse Contents

| Bytes | Contents | Description | Default PG Value |
| --- | --- | --- | --- |
| 00h | 29h | These 2 bytes contain the ID code word for the RTL8812AU. The RTL8812AU will load the contents of the EEPROM into the corresponding location if the ID word (8129h) is correct. | 0x8129 |
| 01h | 81h |  |  |
| 02h | - | Reserved for Realtek. Do not change this field without Realtek’s approval. | 0x02 |
| 03h | - | Reserved for Realtek. Do not change this field without Realtek’s approval. | 0x86 |
| 04h | - | Reserved for Realtek. Do not change this field without Realtek’s approval. | 0x0C |
| 05h | - | Reserved for Realtek. Do not change this field without Realtek’s approval. | 0x00 |
| 06h | - | Reserved for Realtek. Do not change this field without Realtek’s approval. | 0x00 |
| 07h | - | Reserved for Realtek. Do not change this field without Realtek’s approval. | 0x00 |
| 08h | - | Bit[1]: USB3/USB2 transition enable Others: Reserved for Realtek. Do not change this field without Realtek’s approval. | 0x02 |
| 09h~0Fh | RSVD | - | - |
| 10h | Path A 2.4G CCK-1TX Power Index (Absolute Value) | Path A CCK Power Index for Ch 1,2, Range 0~63. | 2Dh |
| 11h |  | Path A CCK Power Index for Ch 3, 4, 5, Range 0~63. | 2Dh |
| 12h |  | Path A CCK Power Index for Ch 6, 7 ,8, Range 0~63. | 2Dh |
| 13h |  | Path A CCK Power Index for Ch 9, 10, 11, Range 0~63. | 2Dh |
| 14h |  | Path A CCK Power Index for Ch 12, 13, Range 0~63. | 2Dh |
| 15h |  | Path A CCK Power Index for Ch 14, Range 0~63. | 2Dh |
| 16h | Path A 2.4G BW40-1S Tx Power Index (Absolute Value) | Path A 2G BW40-1S Power Index for Ch 1, 2, Range 0~63. | 2Dh |
| 17h |  | Path A 2G BW40-1S Power Index for Ch 3, 4, 5, Range 0~63. | 2Dh |
| 18h |  | Path A 2G BW40-1S Power Index for Ch 6, 7 ,8, Range 0~63. | 2Dh |
| 19h |  | Path A 2G BW40-1S Power Index for Ch 9, 10, 11, Range 0~63. | 2Dh |
| 1Ah |  | Path A 2G BW40-1S Power Index for Ch 12, 13, 14 Range 0~63. | 2Dh |
| 1Bh | Path A 2.4G BW20-1S Tx Power Index Difference | Pwower Index Difference between BW20-1S and BW40-1S. Bit[7:4]: Path A 2G Offset, Range –8~7. | 24h |
|  | Path A 2.4G OFDM-1Tx Power Index Difference | Pwower Index Difference between OFDM-1Tx and BW40-1S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 1Ch | Path A 2.4G BW40-2S Tx Power Index Difference | Pwower Index Difference between BW40-2S and BW40-1S. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path A 2.4G BW20-2S Tx Power Index Difference | Pwower Index Difference between BW20-2S and BW20-1S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 1Dh | Path A 2.4G OFDM-2Tx Power Index Difference | Pwower Index Difference between OFDM-2Tx and OFDM-1Tx. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path A 2.4G CCK-2Tx Power Index Difference | Pwower Index Difference between CCK-2T and CCK-1Tx. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 1Eh | Path A 2.4G BW40-3S Tx Power Index Difference | Pwower Index Difference between BW40-3S and BW40-2S. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path A 2.4G BW20-3S Tx Power Index Difference | Pwower Index Difference between BW20-3S and BW20-2S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 1Fh | Path A 2.4G OFDM-3Tx Power Index Difference | Pwower Index Difference between OFDM-3Tx and OFDM-2Tx. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path A 2.4G CCK-3Tx Power Index Difference | Pwower Index Difference between CCK-3T and CCK-2Tx. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 20h | Path A 2.4G BW40-4S Tx Power Index Difference | Pwower Index Difference between BW40-4S and BW40-3S. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path A 2.4G BW20-4S Tx Power Index Difference | Pwower Index Difference between BW20-4S and BW20-3S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 21h | Path A 2.4G OFDM-4Tx Power Index Difference | Pwower Index Difference between OFDM-4Tx and OFDM-3Tx. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path A 2.4G CCK-4Tx Power Index Difference | Pwower Index Difference between CCK-4T and CCK-3Tx. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 22h | Path A 5G BW40-1S Tx Power Index (Absolute Value) | Path A 5G BW40-1S Power Index for Ch 36, 38, 40, Range 0~63. | 2Ah |
| 23h |  | Path A 5G BW40-1S Power Index for Ch 44, 46, 48, Range 0~63. | 2Ah |
| 24h |  | Path A 5G BW40-1S Power Index for Ch 52, 54, 56, Range 0~63. | 2Ah |
| 25h |  | Path A 5G BW40-1S Power Index for Ch 60, 62, 64, Range 0~63. | 2Ah |
| 26h |  | Path A 5G BW40-1S Power Index for Ch 100, 102, 104, Range 0~63. | 2Ah |
| 27h |  | Path A 5G BW40-1S Power Index for Ch 108, 110, 112,Range 0~63. | 2Ah |
| 28h |  | Path A 5G BW40-1S Power Index for Ch 116, 118, 120, Range 0~63. | 2Ah |
| 29h |  | Path A 5G BW40-1S Power Index for Ch 124, 126, 128, Range 0~63. | 2Ah |
| 2Ah |  | Path A 5G BW40-1S Power Index for Ch 132, 134, 136,Range 0~63. | 2Ah |
| 2Bh |  | Path A 5G BW40-1S Power Index for Ch 140, 142, 144,Range 0~63. | 2Ah |
| 2Ch |  | Path A 5G BW40-1S Power Index for Ch 149, 151, 153, Range 0~63. | 2Ah |
| 2Dh |  | Path A 5G BW40-1S Power Index for Ch 157, 159, 161,Range 0~63. | 2Ah |
| 2Eh |  | Path A 5G BW40-1S Power Index for Ch 165, 167, 169, Range 0~63. | 2Ah |
| 2Fh |  | Path A 5G BW40-1S Power Index for Ch 173, 175, 177, Range 0~63. | 2Ah |
| 30h | Path A 5G BW20-1S Tx Power Index Difference | Pwower Index Difference between BW20-1S and BW40-1S. Bit[7:4]: Path A 5G Offset, Range –8~7. | 04h |
|  | Path A 5G OFDM-1Tx Power Index Difference | Pwower Index Difference between OFDM-1Tx and BW40-1S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 31h | Path A 5G BW40-2S Tx Power Index Difference | Pwower Index Difference between BW40-2S and BW40-1S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G BW20-2S Tx Power Index Difference | Pwower Index Difference between BW20-2S and BW20-1S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 32h | Path A 5G BW40-3S Tx Power Index Difference | Pwower Index Difference between BW40-3S and BW40-2S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G BW20-3S Tx Power Index Difference | Pwower Index Difference between BW20-3S and BW20-2S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 33h | Path A 5G BW40-4S Tx Power Index Difference | Pwower Index Difference between BW40-4S and BW40-3S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G BW20-4S Tx Power Index Difference | Pwower Index Difference between BW20-4S and BW20-3S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 34h | Path A 5G OFDM-2Tx Power Index Difference | Pwower Index Difference between OFDM-2T and OFDM-1Tx. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G OFDM-3Tx Power Index Difference | Pwower Index Difference between OFDM-3T and OFDM-2Tx. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 35h | Path A 5G OFDM-4Tx Power Index Difference | Bit[7:4]: Resever. Pwower Index Difference between OFDM-4T and OFDM-3Tx. Bit[3:0]: Path A 5G Offset, Range –8~7. | EEh |
| 36h | Path A 5G BW80-1S Tx Power Index Difference | Pwower Index Difference between BW80-1S and BW40-1S (UpSide Ch + LowSide Ch)/2. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G BW160-1S Tx Power Index Difference | Pwower Index Difference between BW160-1S and BW80-1S (UpSide Ch + LowSide Ch)/2. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 37h | Path A 5G BW80-2S Tx Power Index Difference | Pwower Index Difference between BW80-2S and BW80-1S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G BW160-2S Tx Power Index Difference | Pwower Index Difference between BW160-2S and BW160-1S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 38h | Path A 5G BW80-3S Tx Power Index Difference | Pwower Index Difference between BW80-3S and BW80-2S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G BW160-3S Tx Power Index Difference | Pwower Index Difference between BW160-3S and BW160-2S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 39h | Path A 5G BW80-4S Tx Power Index Difference | Pwower Index Difference between BW80-4S and BW80-3S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path A 5G BW160-4S Tx Power Index Difference | Pwower Index Difference between BW160-4S and BW160-3S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 3Ah | Path B 2.4G CCK-1TX Power Index (Absolute Value) | Path B CCK Power Index for Ch 1,2, Range 0~63. | 2Dh |
| 3Bh |  | Path B CCK Power Index for Ch 3, 4, 5, Range 0~63. | 2Dh |
| 3Ch |  | Path B CCK Power Index for Ch 6, 7 ,8, Range 0~63. | 2Dh |
| 3Dh |  | Path B CCK Power Index for Ch 9, 10, 11, Range 0~63. | 2Dh |
| 3Eh |  | Path B CCK Power Index for Ch 12, 13, Range 0~63. | 2Dh |
| 3Fh |  | Path B CCK Power Index for Ch 14, Range 0~63. | 2Dh |
| 40h | Path A 2.4G BW40-1S Tx Power Index (Absolute Value) | Path A 2G BW40-1S Power Index for Ch 1, 2, Range 0~63. | 2Dh |
| 41h |  | Path A 2G BW40-1S Power Index for Ch 3, 4, 5, Range 0~63. | 2Dh |
| 42h |  | Path A 2G BW40-1S Power Index for Ch 6, 7 ,8, Range 0~63. | 2Dh |
| 43h |  | Path A 2G BW40-1S Power Index for Ch 9, 10, 11, Range 0~63. | 2Dh |
| 44h |  | Path A 2G BW40-1S Power Index for Ch 12, 13, 14 Range 0~63. | 2Dh |
| 45h | Path B 2.4G BW20-1S Tx Power Index Difference | Pwower Index Difference between BW20-1S and BW40-1S. Bit[7:4]: Path A 2G Offset, Range –8~7. | 24h |
|  | Path B 2.4G OFDM-1Tx Power Index Difference | Pwower Index Difference between OFDM-1Tx and BW40-1S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 46h | Path B 2.4G BW40-2S Tx Power Index Difference | Pwower Index Difference between BW40-2S and BW40-1S. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path B 2.4G BW20-2S Tx Power Index Difference | Pwower Index Difference between BW20-2S and BW20-1S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 47h | Path B 2.4G OFDM-2Tx Power Index Difference | Pwower Index Difference between OFDM-2Tx and OFDM-1Tx. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path B 2.4G CCK-2Tx Power Index Difference | Pwower Index Difference between CCK-2T and CCK-1Tx. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 48h | Path B 2.4G BW40-3S Tx Power Index Difference | Pwower Index Difference between BW40-3S and BW40-2S. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path B 2.4G BW20-3S Tx Power Index Difference | Pwower Index Difference between BW20-3S and BW20-2S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 49h | Path B 2.4G OFDM-3Tx Power Index Difference | Pwower Index Difference between OFDM-3Tx and OFDM-2Tx. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | PathB 2.4G CCK-3Tx Power Index Difference | Pwower Index Difference between CCK-3T and CCK-2Tx. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 4Ah | Path B 2.4G BW40-4S Tx Power Index Difference | Pwower Index Difference between BW40-4S and BW40-3S. Bit[7:4]: Path A 2G Offset, Range –8~7. | EEh |
|  | Path B 2.4G BW20-4S Tx Power Index Difference | Pwower Index Difference between BW20-4S and BW20-3S. Bit[3:0]: Path A 2G Offset, Range –8~7. |  |
| 4Bh | Path B 2.4G OFDM-4Tx Power Index Difference | Pwower Index Difference between OFDM-4Tx and OFDM-3Tx. Bit[7:4]: Path B 2G Offset, Range –8~7. | EEh |
|  | Path B 2.4G CCK-4Tx Power Index Difference | Pwower Index Difference between CCK-4T and CCK-3Tx. Bit[3:0]: Path B 2G Offset, Range –8~7. |  |
| 4Ch | Path B 5G BW40-1S Tx Power Index (Absolute Value) | Path A 5G BW40-1S Power Index for Ch 36, 38, 40, Range 0~63. | 2Ah |
| 4Dh |  | Path A 5G BW40-1S Power Index for Ch 44, 46, 48, Range 0~63. | 2Ah |
| 4Eh |  | Path A 5G BW40-1S Power Index for Ch 52, 54, 56, Range 0~63. | 2Ah |
| 4Fh |  | Path A 5G BW40-1S Power Index for Ch 60, 62, 64, Range 0~63. | 2Ah |
| 50h |  | Path A 5G BW40-1S Power Index for Ch 100, 102, 104, Range 0~63. | 2Ah |
| 51h |  | Path A 5G BW40-1S Power Index for Ch 108, 110, 112,Range 0~63. | 2Ah |
| 52h |  | Path A 5G BW40-1S Power Index for Ch 116, 118, 120, Range 0~63. | 2Ah |
| 53h |  | Path A 5G BW40-1S Power Index for Ch 124, 126, 128, Range 0~63. | 2Ah |
| 54h |  | Path A 5G BW40-1S Power Index for Ch 132, 134, 136,Range 0~63. | 2Ah |
| 55h |  | Path A 5G BW40-1S Power Index for Ch 140, 142, 144,Range 0~63. | 2Ah |
| 56h |  | Path A 5G BW40-1S Power Index for Ch 149, 151, 153, Range 0~63. | 2Ah |
| 57h |  | Path A 5G BW40-1S Power Index for Ch 157, 159, 161,Range 0~63. | 2Ah |
| 58h |  | Path A 5G BW40-1S Power Index for Ch 165, 167, 169, Range 0~63. | 2Ah |
| 59h |  | Path A 5G BW40-1S Power Index for Ch 173, 175, 177, Range 0~63. | 2Ah |
| 5Ah | Path B 5G BW20-1S Tx Power Index Difference | Pwower Index Difference between BW20-1S and BW40-1S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G OFDM-1Tx Power Index Difference | Pwower Index Difference between OFDM-1Tx and BW40-1S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 5Bh | Path B 5G BW40-2S Tx Power Index Difference | Pwower Index Difference between BW40-2S and BW40-1S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G BW20-2S Tx Power Index Difference | Pwower Index Difference between BW20-2S and BW20-1S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 5Ch | Path B 5G BW40-3S Tx Power Index Difference | Pwower Index Difference between BW40-3S and BW40-2S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G BW20-3S Tx Power Index Difference | Pwower Index Difference between BW20-3S and BW20-2S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 5Dh | Path B 5G BW40-4S Tx Power Index Difference | Pwower Index Difference between BW40-4S and BW40-3S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G BW20-4S Tx Power Index Difference | Pwower Index Difference between BW20-4S and BW20-3S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 5Eh | Path A 5G OFDM-2Tx Power Index Difference | Pwower Index Difference between OFDM-2T and OFDM-1Tx. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G OFDM-3Tx Power Index Difference | Pwower Index Difference between OFDM-3T and OFDM-2Tx. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 5Fh | Path B 5G OFDM-4Tx Power Index Difference | Bit[7:4]: Resever. Pwower Index Difference between OFDM-4T and OFDM-3Tx. Bit[3:0]: Path A 5G Offset, Range –8~7. | EEh |
| 60h | Path B 5G BW80-1S Tx Power Index Difference | Pwower Index Difference between BW80-1S and BW40-1S (UpSide Ch + LowSide Ch)/2. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G BW160-1S Tx Power Index Difference | Pwower Index Difference between BW160-1S and BW80-1S (UpSide Ch + LowSide Ch)/2. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 61h | Path B 5G BW80-2S Tx Power Index Difference | Pwower Index Difference between BW80-2S and BW80-1S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G BW160-2S Tx Power Index Difference | Pwower Index Difference between BW160-2S and BW160-1S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 62h | Path B 5G BW80-3S Tx Power Index Difference | Pwower Index Difference between BW80-3S and BW80-2S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G BW160-3S Tx Power Index Difference | Pwower Index Difference between BW160-3S and BW160-2S. Bit[3:0]: Path A 5G Offset, Range –8~7. |  |
| 63h | Path B 5G BW80-4S Tx Power Index Difference | Pwower Index Difference between BW80-4S and BW80-3S. Bit[7:4]: Path A 5G Offset, Range –8~7. | EEh |
|  | Path B 5G BW160-4S Tx Power Index Difference | Pwower Index Difference between BW160-4S and BW160-3S. Bit[3:0]: Path B 5G Offset, Range –8~7. |  |
| 64h~B7h | RSVD | - | - |
| B8h | Channel Plan | Bit[7]: Software configure mode 0h: Enable software configure( refer to Channel Plane Domain Code) 1h: Disable software configure( can't change Channel Plan Setting) Bit[6:0]: Channel Plan Country Code; Bit[6:0]; channels. 2G RD; 5G RD; 2G; 5G. Worldwide 13; 5G_NULL; 20h; 1~13; NA. Europe 2G; 5G_NULL; 21h; 1~13; NA. US 2G; 5G_NULL; 22h; 1~11; NA. Japan 2G; 5G_NULL; 23h; 1~13,14; NA. France 2G; 5G_NULL; 24h; 10~13; NA. US_2G; US_5G; 25h; 1~11; 36~48, 52~64, 100~140, 149~165. Worldwide 13; Europe 5G; 26h; 1~13; 36~48, 52~64, 100~140. Japan 2G; Japan 5G; 27h; 1~13,14; 36~48, 52~64, 100~140. Worldwide 13; Korea 5G; 28h; 1~13; 36~48, 52~64, 100~124, 149~165. Worldwide 13; US w/o DFS Channels; 29h; 1~13; 36~48, 149~165. Worldwide 13; India, Mexico; 30h; 1~13; 36~48, 52~64, 149~165. Worldwide 13; Venezuela; 31h; 1~13; 36~48, 52~64, 149~161. Worldwide 13; China; 32h; 1~13; 149~165. Worldwide 13; Israel; 33h; 1~13; 36~48, 52~64. US 2G; US/Canada; 34h; 1~11; 36~48, 52~64, 100~116, 136, 140, 149~165. Worldwide 13; Australia, New Zealand; 35h; 1~13; 36~48, 52~64, 100~140, 149~165. Worldwide 13; Russia; 36h; 1~13; 36~48, 52~64, 100~132, 149~165. Japan 2G; Japan(W52, W53); 37h; 1~13,14; 36~48, 52~64. Japan 2G; Japan (W56); 38h; 1~13,14; 100~140. US 2G; Taiwan; 39h; 1~11; 56~64, 100~116, 136, 140, 149~165. US 2G; Taiwan w/o DFS; 40h; 1~11; 56~64, 149~165. Global domain; 5G_NULL; 41h; 1~13,14; NA. Europe 2G; Europe 5G w/o DFS; 42h; 1~13; 36~48. US 2G; US w/o DFS Channels; 43h; 1~11; 36~48, 149~165. US 2G; Taiwan 5G w/o DFS, band4 only; 44h; 1~11; 149~165. | 7Fh |
|  |  | Country Code; Bit[6:0]; channels. 2G RD; 5G RD; 2G; 5G. Worldwide 13; Australia, New Zealand; 45h; Ch1~13; Ch36~48, Ch52~64, Ch100~116, Ch136, Ch140, Ch149~165 (o/w Weather radar). US 2G; Latin America; 46h; 1~11; Ch149~161. Worldwide 13; US_5G; 7Fh; 1~13; 36~48, 52~64, 100~140, 149~165. |  |
| B9h | Crystal Calibration | XTAL_K Value Bit[5:0], Xi=Xo Range 0~3F h. Bit[7:6]: reserved FF h = 00 h | 20h |
| BAh | Thermal Meter | Thermal Meter Default Value System maker will calibrate a value and save it in EEPROM. Bit[7:0]: Thermal Meter Value. FF h: disable Tx power tracking function | FFh |
| BBh | IQ Calibration and LC Calibration | [3:2] : Do LCK by Thermal Meter △value. 0h : △4 (~25degC) 1h : △3 (~20degC) 2h : △2 (~10degC) 3h : don’t LCK. [1:0] : Do IQK by Thermal Meter △value. 0h : △5 (~30degC) 1h : △3 (~20degC) 2h : △2 (~10degC) 3h : don’t IQK. | 00h |
| BCh | 2G and 5G PA Type | 2G PA Bit[7]: Path-D Internal/External PA 0h: Internal PA 1 h: External PA Bit[6]: Path-C Internal/External PA 0h: Internal PA 1h: External PA Bit[5]: Path-B Internal/External PA 0h: Internal PA 1h: External PA Bit[4]: Path-A Internal/External PA 0h: Internal PA 1h: External PA 5G PA Bit[3]: Path-D Internal/External PA 0h: Internal PA 1 h: External PA Bit[2]: Path-C Internal/External PA 0h: Internal PA 1h: External PA Bit[1]: Path-B Internal/External PA 0h: Internal PA 1h: External PA Bit[0]: Path-A Internal/External PA 0h: Internal PA 1h: External PA | 00h |
| BDh | 2G LNA Type and Gain Selection | Bit[2:0]: 2G path-A external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[3]: 2G Path-A Internal/External LNA 0h: Internal LNA 1h: External LNA Bit[6:4]: 2G path-B external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[7]: 2G Path-B Internal/External LNA 0h: Internal LNA 1h: External LNA | 00h |
| BE |  | Bit[2:0]: 2G path-C external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[3]: 2G Path-C Internal/External LNA 0h: Internal LNA 1h: External LNA Bit[6:4]: 2G path-D external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[7]: 2G Path-D Internal/External LNA 0h: Internal LNA 1h: External LNA | 00h |
| BFh | 5G LNA Type and Gain Selection | Bit[2:0]: 5G path-A external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[3]: 5G Path-A Internal/External LNA 0h: Internal LNA 1h: External LNA Bit[6:4]: 5G path-B external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[7]: 5G Path-B Internal/External LNA 0h: Internal LNA 1h: External LNA | 00h |
| C0h |  | Bit[2:0]: 5G path-C external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[3]: 5G Path-C Internal/External LNA 0h: Internal LNA 1h: External LNA Bit[6:4]: 5G path-Dexternal LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[7]: 5G Path-D Internal/External LNA 0h: Internal LNA 1h: External LNA | 00h |
| C1h | Board Options | Bit[2:0]: Regulatory selection. 0h: driver-defined maximum power offset for longer communication range. ( refer to Power by rate table) 1h: Power limit table-defined maximum power offset range ( refer to Power by rate table and Power limit table to take the smaler index value) 2h: not support power offset by rate (Don't refer to Power by rate table) 3h~7h: reserved Bit[3]: Non-interrupt Antenna Diversity 0: disable 1: enable Bit[4]: reserved Bit[7:5]: Board Type (PCIe) 0h: WiFi solo-mCard 1h: WiFi+BT combo-mCard 2h: PCIe Card 3h~31h: reserved. | 00h |
| C2h | Feature Options | Bit[1:0]: function configuration of pin_LED0 and pin_LED1 Bit[3:2]: Link Speed shown in OS 0h: Current Tx PHY Rate 1h: Current Rx PHY Rate 2h: Maximum RX PHY Rate 3h: reserved Bit[4]: power down mode selection 0: radio off 1: power down Bit[5]: Enable bluetooth coexistence 0: Disable 1: Enable Bit[6]: Enable WoWLAN 0: Disable 1: Enable Bit[7]: Enable WAPI support 0: Disable 1: Enable | 00h |
| C3h | BT Setting | Bit[0]: Total antenna number 0: 2-Antenna (default) 1: 1-Antenna Bit[3:1]: Co-existence type 0h: 2-wire type (default) 1h: ISSC 3-wire type 2h: Accel type 3h: CSR traditional type 4h: CSR enhance type 5h: RTL8756 type 6h~8h: reserved Bit[4]: antenna isolation quality 0:good, 1:bad(default) Bit[5]: Radio on/off type 0: combine with WiFi, 1:individual Bit[7:6]: reserved | 00h |
| C4h | The EEPROM content version. | The Efuse Contents version. | 00h |
| C5h | Customer ID | Customer ID (0x00 and 0xFF are reserved for Realtek) | 00h |
| C6h | 2G Tx BB Swing Setting | Bit[1:0]: 2G PathA OFDM 0h: 0dB (default) 1h: -3dB 2h: -6dB 3h: -9dB Bit[3:2]: 2G PathB OFDM 0h: 0dB (default) 1h: -3dB 2h: -6dB 3h: -9dB Bit[5:4]: 2G PathC OFDM 0h: 0dB (default) 1h: -3dB 2h: -6dB 3h: -9dB Bit[7:6]: 2G PathD OFDM 0h: 0dB (default) 1h: -3dB 2h: -6dB 3h: -9dB | 00h |
| C7h | 5G Tx BB Swing Setting | Bit[1:0]: 5G PathA OFDM 0h: 0dB 1h: -3dB (default) 2h: -6dB 3h: -9dB Bit[3:2]: 5G PathB OFDM 0h: 0dB 1h: -3dB (default) 2h: -6dB 3h: -9dB Bit[5:4]: 5G PathC OFDM 0h: 0dB 1h: -3dB (default) 2h: -6dB 3h: -9dB Bit[7:6]: 5G PathD OFDM 0h: 0dB 1h: -3dB (default) 2h: -6dB 3h: -9dB | 55h |
| C8h | Tx Power Calibratior Rate | Bit[0]: 2G 40M Tx Power Calibrator Rate. 0h : HT40 (default) 1h : VHT40 Bit[0]: 5G 40M Tx Power Calibrator Rate. 0h : HT40 (default) 1h : VHT40 Bit[7:2]: reserved | 00h |
| C9h | T/Rx Antenna Options | TBD | 00h |
| CBh~CFh | RSVD | - | - |
| D0h | USB_VID | USB Vender ID | DAh |
| D1h |  |  | 0Bh |
| D2h | USB_PID | USB Product ID | 12h |
| D3h |  |  | 88h |
| D4h | USB_Option_0 | Bit[7]: disable FS qualifier discripter 1b : disable 0b : enable Bit[6:3]: Reserved for Realtek. Do not change the value without the approval from Realtek. Bit[2] : Bus Power 0 : bus power 1 : self power Bit[1] : Remote Wakeup 0 : do not support 1 : support Bit[0] : Serial Number 1 : Response from USB string descriptor 3 0 : Response from Efuse offset DCh~D7h | 41h |
| D5h | USB_Option_1 | Bit[7:5] : Reserved for Realtek. Do not change the value without the approval from Realtek. Bit[4] : LTM Support (0 : Disable, 1 : Enable) Bit[3:1] : Reserved for Realtek. Do not change the value without the approval from Realtek. Bit[0]: Link Power Management(LPM) support (0 : Disable, 1 : Enable) This field is valid in USB High Speed mode. While 1: enable the capability and the bcdUSB = 210h, support USB 2.0 extendtion descriptor Wiile 0: do not support this capability, bcdUSB = 200h, and response STALL for USB 2.0 extendtion descriptor request. | 66h |
| D6h | USB_Option_2 | Bit[7:4] : Reserved for Realtek. Do not change the value without the approval from Realtek. Bit[3] BESL enable Bit[2:0] : Reserved for Realtek. Do not change the value without the approval from Realtek. | 42h |
| D7h~DCh | MAC Address | MAC Address |  |
| DDh~116h | USB_String | Manufacture String and Product String |  |
| 117h | USB_Option_3 | Bit[7:6]: RSVD Bit[5] enable rxeleidle poll in U3 Bit[4] recovery idle 1ms timeout enable Bit[3:0] bInterval value in endpoint descriptor. Interval for servicing the interrupt endpoint for data transfers. Expressed in 125us units. | 21h |
| 118h | USB_Option_4 | Bit[7:4]: RSVD bit[3:0] bMaxBurst value in companion descriptor. The maximum number of packets the endpoint can send as part of a burst. | 03h |
| 119h | USB_Option_5 | Bit[7:4]: RSVD bit[3:0] bMaxBurst value in companion descriptor. The maximum number of packets the endpoint can receive as part of a burst. | 03h |
| 11Ah | bU1DevExitLat | bU1DevExitLat value in SuperSpeed Device Capabilities Descriptor. U1 Device Exit Latency. |  |
| 11Bh~11Ch | bU2DevExitLat | bU2DevExitLat value in SuperSpeed Device Capabilities Descriptor. U2 Device Exit Latency. |  |
| 11Dh~12Fh | RSVD | - | - |
| 130h | Uphy_Parameter | USB phy parameters Do not change the value without the approval by Realtek. | 0x81 |
| 131h |  |  | 0xAE |
| 132h |  |  | 0x98 |
| 133h |  |  | 0x2D |
| 134h |  |  | 0x03 |
| 135h |  |  | 0x93 |
| 136h |  |  | 0x96 |
| 137h |  |  | 0x11 |
| 138h |  |  | 0xFC |
| 139h |  |  | 0x8C |
| 13Ah |  |  | 0x00 |
| 13Bh |  |  | 0x11 |
| 13Ch |  |  | 0x9B |
| 13Dh |  |  | 0x78 |
| 13Eh |  |  | 0x4A |
| 13Fh |  |  | 0xCE |
| 140h |  |  | 0xE0 |
| 141h |  |  | 0x48 |
| 142h |  |  | 0xE0 |
| 143h |  |  | 0x70 |
| 144h |  |  | 0x27 |
| 145h |  |  | 0x00 |
| 146h |  |  | 0x78 |
| 147h |  |  | 0xEA |
| 148h |  |  | 0x60 |
| 149h |  |  | 0xD0 |
| 14Ah |  |  | 0xE1 |
| 14Bh |  |  | 0x20 |
| 14Ch |  |  | 0x32 |
| 14Dh |  |  | 0x61 |
| 14Eh |  |  | 0x4F |
| 14Fh |  |  | 0x3C |
| 150h |  |  | 0x92 |
| 151h |  |  | 0x40 |
| 152h |  |  | 0x92 |
| 153h |  |  | 0x15 |
| 154h |  |  | 0x8B |
| 155h |  |  | 0x6A |
| 156h |  |  | 0x4D |
| 157h |  |  | 0x8A |
| 158h |  |  | 0x17 |
| 159h |  |  | 0xC1 |
| 15Ah |  |  | 0x98 |
| 15Bh |  |  | 0x00 |
| 15Ch |  |  | 0x80 |
| 15Dh |  |  | 0x0C |
| 15Eh |  |  | 0x00 |
| 15Fh |  |  | 0x4C |
| 160h |  |  | 0xFC |
| 161h |  |  | 0x81 |
| 162h |  |  | 0x0C |
| 163h |  |  | 0x01 |
| 164h |  |  | 0xDE |
| 165h |  |  | 0x00 |
| 166h |  |  | 0x00 |
| 167h |  |  | 0x00 |
| 168h |  |  | 0x00 |
| 169h |  |  | 0x02 |
| 16Ah |  |  | 0xE1 |
| 16Bh |  |  | 0x63 |
| 16Ch |  |  | 0x12 |
| 16Dh |  |  | 0xDD |
| 16Eh |  |  | 0x21 |
| 16Fh |  |  | 0x00 |
| 170h |  |  | 0xCB |
| 171h |  |  | 0x3F |
| 172h |  |  | 0xA0 |
| 173h |  |  | 0xE0 |
| 174h |  |  | 0xC2 |
| 175h |  |  | 0xF0 |
| 176h |  |  | 0xA3 |
| 177h |  |  | 0x4F |
| 178h |  |  | 0x04 |
| 179h |  |  | 0x36 |
| 17Ah |  |  | 0x08 |
| 17Bh |  |  | 0x41 |
| 17Ch |  |  | 0x05 |
| 17Dh |  |  | 0xEB |
| 17Eh |  |  | 0x94 |
| 17Fh |  |  | 0xD7 |
| 180h |  |  | 0x14 |
| 181h |  |  | 0x6D |
| 182h |  |  | 0xC8 |
| 183h |  |  | 0x67 |
| 184h |  |  | 0x80 |
| 185h |  |  | 0x20 |
| 186h |  |  | 0xF8 |
| 187h |  |  | 0x80 |
| 188h |  |  | 0x30 |
| 189h |  |  | 0x80 |
| 18Ah |  |  | 0x30 |
| 18Bh |  |  | 0x38 |
| 18Ch |  |  | 0x20 |
| 18Dh |  |  | 0x34 |
| 18Eh |  |  | 0xA1 |
| 18Fh |  |  | 0x94 |
| 190h |  |  | 0x77 |
| 191h |  |  | 0xB2 |
| 192h |  |  | 0x94 |
| 193h~1FFh | RSVD | Reserved for Realtek. Do not change the value without the approval from Realtek. |  |